SCOUT CoinScan — Jetson ↔ Pico Command Protocol

v0.1 · USB CDC serial · 115200 baud 8N1 · ASCII lines terminated \n

The Jetson (host) is the master and issues commands; the RP2040 Pico (controller) executes real-time motion, lighting, and sensor tasks and replies. Fields are space-separated. Every command carries an integer sequence id so replies can be matched.

# Line format

Host → Pico:   <seq> <CMD> [args...]

Pico → Host:   <seq> OK [data...]          // success

<seq> ERR <code> <msg>      // failure

EVT <name> [data...]        // asynchronous, no seq

The Pico echoes the seq it received. Events (EVT) are unsolicited — beam breaks, faults, interlock trips — and carry no seq.

# Commands

| CMD | Args | Action | Reply |
| --- | --- | --- | --- |
| PING | — | Liveness check | OK <fw_version> |
| HOME | — | Home Z-stage, center V-stop, paddles flat | OK when settled |
| FEED | — | Index singulation disk 90°, present next coin | OK or ERR JAM |
| ZMOVE | <µm> | Absolute Z-stage move (0 = platen focus datum) | OK <pos_um> |
| ZSWEEP | <start> <step> <n> | Move + EVT FOCUSPT at each stop for AF | OK after last |
| LIGHT | <ch> <pct> | Set one lighting channel | OK |
| CAPSEQ | <preset> | Run stored light sequence; EVT FRAME per state | OK after sequence |
| LASER | <0¦1> | Line laser off/on (interlock-gated) | OK or ERR INTERLOCK |
| LSWEEP | <deg0> <deg1> <ms> | Pivot laser across coin; EVT LASERPT per step | OK |
| VSTOP | <0¦1> | 0 = hold coin, 1 = retract/release | OK |
| GATE | <n> <0¦1> | Paddle n (1–8): 0 flat, 1 deflect | OK |
| SORT | <bin> | Set paddle, retract V-stop, watch rail-end beam | OK <bin> or ERR MISS |
| STATUS | — | Dump sensor + position state | OK <json> |
| ESTOP | — | All motion halt, laser+UV off, paddles flat | OK |

# Lighting channels (LIGHT)

0 white ring · 1 UV 365 nm · 2 blue 470 nm · 3 red 630 nm · 4 IR 850 nm · 5..8 white quadrants N/E/S/W (photometric stereo). pct = 0–100.

# Capture presets (CAPSEQ)

- 1 FAST — cross-polarized white only (bulk sorting)

- 2 FULL — white → quad N/E/S/W → blue → red → IR → UV, EVT FRAME at each (deep scan)

- 3 CALIB — white flat field for px/mm + white balance

# Events

| EVT | Data | Meaning |
| --- | --- | --- |
| BEAM | <id> <0¦1> | Break-beam edge (1 entry, 2 pre-gate, 3 rail-end) |
| FOCUSPT | <pos_um> | Z-stage settled at sweep stop; host computes sharpness |
| FRAME | <tag> | Lights set for <tag>; host grabs both cameras now |
| LASERPT | <deg> | Laser at angle; host grabs top-cam frame |
| MISS | <bin> | Rail-end beam fired after SORT — coin missed its gate |
| JAM | <where> | Disk stall or beam-timeout |
| INTERLOCK | <0¦1> | Lid opened (1) / closed (0); laser+UV force-off on open |
| FAULT | <code> | Driver fault (stepper OT, servo brownout) |

# Typical single-coin cycle (host orchestration)

- FEED — coin lands; EVT BEAM 1 1 fires when it reaches the station

- VSTOP 0 (hold) — coin self-centers on the V

- ZSWEEP → collect EVT FOCUSPT, host finds peak, ZMOVE <peak>

- CAPSEQ 2 FULL → host grabs frame pairs on each EVT FRAME

- LSWEEP → host builds height map from EVT LASERPT frames

- Host runs vision → decides bin

- SORT <bin> → V-stop retracts, coin routes; ERR MISS if rail-end beam trips

- Paddle auto-returns flat; loop to 1 (next coin already staged)

# Safety invariants (enforced on the Pico, not the host)

- LASER 1 and UV (LIGHT 1 >0) are refused with ERR INTERLOCK whenever the lid switch is open; both are hardware-series-cut regardless.

- Any EVT INTERLOCK 1 immediately zeroes laser + UV channels on the Pico without waiting for the host.

- Watchdog: if no host line for 3 s during motion, the Pico executes ESTOP.

Design intent: the Pico guarantees timing and safety; the Jetson owns all vision and decisions. Keeping the split this clean is what lets each half be released and reused independently.